# RA8P1 婴儿监护项目自动实测报告（测试111）

# 1. 结论与范围

生成时间：2026-07-28T01:16:40+08:00；Git 基线：7ba3c09；RA8/视觉 build：0x20260728/0x20260728；网关固件：esp32s3cam-ra8-gateway-3.0.3。

本报告只填写实际读取、实际下发或现有带标签测试报告能够证明的数据。需要玩偶、标准配重、参考温度源、遮挡、断网等外部条件的项目保留为待测试。

当前自动结果：通过 24 项，部分通过 2 项，发现故障 0 项，未测试 5 项。

关键结论：烧录后视觉持续自动触发，配置的最小调度间隔为 1000 ms；受串口抓图和处理链路限制，本次实测为 0.188 fps（约 5.32 秒/帧）。录音与拍照只在命令触发时写 SD；麦克风瞬态 I2S 错误已增加自动恢复；热成像当前连续 20/20 组完整有效帧，当前已测部件均正常。

# 2. 自动测试总表

| 编号 | 项目 | 状态 | 实测证据 | 后续动作 |
| --- | --- | --- | --- | --- |
| B01 | CPU0 固件构建 | 通过 | text=5945162, data=144, bss=8098792 bytes |  |
| B02 | CPU1 固件构建 | 通过 | text=87158, data=116, bss=327308 bytes |  |
| B03 | 双核烧录 | 通过 | J-Link Program/Verify 成功；CPU1 内容匹配 |  |
| S01 | 服务端单元测试 | 通过 | 39/39 |  |
| S02 | 音频训练流水线测试 | 通过 | 22/22；1 条弃用警告 |  |
| S03 | 视觉训练流水线测试 | 通过 | 21/21 |  |
| S04 | RA8/ESP32 协议合约测试 | 通过 | 4/4 |  |
| H01 | 烧录后持续视觉检测 | 通过 | 连续 50 帧，frame 41..90，实际 0.188 fps（5320 ms/帧），JPEG/NPU/drop=0/0/0 |  |
| H02 | 自动推理不常驻写 SD | 通过 | camera_saved_delta=0，auto=1，最小调度间隔 1000 ms；实测平均帧间隔 5320 ms |  |
| H03 | 三视觉模型串行时延 | 通过 | 总耗时 avg/P95=354.0/354.0 ms；scene/occlusion/behavior=159.0/29.0/29.0 ms |  |
| H04 | 服务器到 RA8 命令闭环 | 通过 | 6/6；含播放、录音、拍照和后台持续性 |  |
| H05 | 麦克风录制后继续检测 | 通过 | {'microphone_status': 2, 'microphone_last_fsp_error': 0, 'microphone_read_delta': 322, 'cry_inference_delta': 16, 'camera_frame_delta': 1, 'vision_inference_delta': 1} |  |
| H06 | 一级哭声与视觉并行运行 | 通过 | 哭声推理 50 帧，avg/P95=31.0/31 ms，mic read delta=2710，同时 vision delta=6，异常状态样本=0 |  |
| H07 | 热成像连续完整帧 | 通过 | valid samples=20/20, sequence=178554665->178554731, bytes=10248, pixels=5120, CRC=26863/26863, UART errors=114->114 |  |
| D01 | ra8_link | 通过 | RA8 online=1, status age=418 ms |  |
| D02 | sd_card | 通过 | SD ready=1, step=7, failure=0 |  |
| D03 | thermal | 通过 | valid samples=20/20, sequence=178554665->178554731, bytes=10248, pixels=5120, CRC=26863/26863, UART errors=114->114 |  |
| D04 | bed_temperature | 通过 | TMP117 connected=0x1F, errors=0x00 |  |
| D05 | pressure | 通过 | pressure state=3, last error=8 |  |
| D06 | microphone | 通过 | mic state=2, read=638137->640122, cumulative errors=3->3 |  |
| D07 | camera | 通过 | camera state=5, probe=1, saved=3, writer error=0 |  |
| M01 | 一级哭声 float 测试集 | 通过 | n=10784，cry P/R/F1=80.80%/93.64%/86.74% |  |
| M02 | 一级哭声 INT8 测试集 | 通过 | n=10784，macro P/R/F1=89.26%/92.92%/90.82% |  |
| M03 | 二级哭声 INT8 五类 | 通过 | n=1018，macro P/R/F1=70.68%/72.13%/71.13% |  |
| M04 | 二级哭声 unknown 拒识 | 部分通过 | recordings=211，coverage=76.30%，accepted accuracy=93.79%，unknown=50 | 补充真实 unknown 类标注样本后才能计算 unknown precision/recall/F1 |
| M05 | 场景检测指标 | 未测试 | 当前仓库没有与已部署模型严格匹配的带标签测试报告；不能用连续运行代替准确率 |  |
| M06 | 遮盖分类指标 | 未测试 | 当前仓库没有与已部署模型严格匹配的带标签测试报告；不能用连续运行代替准确率 |  |
| M07 | 行为分类指标 | 未测试 | 当前仓库没有与已部署模型严格匹配的带标签测试报告；不能用连续运行代替准确率 |  |
| F01 | 正常/边界/危险事件各 20 次 | 未测试 | 未使用模板中的示例 20/20；当前未施加玩偶、配重、遮挡和标准音频时间线 |  |
| X01 | 热成像历史故障诊断 | 部分通过 | 早先曾捕获 thermal=fault；当前已恢复，连续 20/20 组完整有效帧。未执行受控故障注入和恢复计时 |  |
| X02 | 摄像头/压力/SD/网络故障注入 | 未测试 | 未执行拔线、写满、断网和服务器重启，避免将未执行项写成通过 |  |

# 3. 当前部件诊断

| 部件 | 状态 | 摘要 | 详情 |
| --- | --- | --- | --- |
| ra8_link | pass | 通过 | RA8 online=1, status age=418 ms |
| sd_card | pass | 通过 | SD ready=1, step=7, failure=0 |
| thermal | pass | 通过 | valid samples=20/20, sequence=178554665->178554731, bytes=10248, pixels=5120, CRC=26863/26863, UART errors=114->114 |
| bed_temperature | pass | 通过 | TMP117 connected=0x1F, errors=0x00 |
| pressure | pass | 通过 | pressure state=3, last error=8 |
| microphone | pass | 通过 | mic state=2, read=638137->640122, cumulative errors=3->3 |
| camera | pass | 通过 | camera state=5, probe=1, saved=3, writer error=0 |

# 4. 命令闭环实测

| 动作 | 结果 | 网关状态 | ACK/ms | 板端确认/ms | 板端证据 |
| --- | --- | --- | --- | --- | --- |
| device.refresh | 通过 | acked | 672.0 | 78.0 | {"telemetry_sequence":15992} |
| audio.play_boot | 通过 | acked | 407.0 | 3406.0 | {"cmd_seq":3,"ack":3,"state":0,"file":"BOOT.WAV","elapsed_ms":600} |
| microphone.record.start | 通过 | acked | 359.0 | 1906.0 | {"cmd_seq":7,"ack":7,"record_active":1,"writer_state":7,"writer_error":0,"file":"T005330.WAV"} |
| microphone.record.stop | 通过 | acked | 375.0 | 3062.0 | {"cmd_seq":8,"ack":8,"record_active":0,"writer_state":3,"writer_error":0,"writer_bytes":147456,"file":"T005330.WAV"} |
| camera.capture | 通过 | acked | 437.0 | 10047.0 | {"cmd_seq":3,"ack":3,"saved":3,"writer_error":0,"writer_bytes":45104,"file":"T005339.JPG","soi":1,"eoi":0} |
| post_command.continuity | 通过 | not_applicable | 0.0 | 4579.0 | {"microphone_status":2,"microphone_last_fsp_error":0,"microphone_read_delta":322,"cry_inference_delta":16,"camera_frame_delta":1,"vision_inference_delta":1} |

# 5. 单模态模型结果

| 模型 | 关键类别 | 样本数 | Precision | Recall | F1 |
| --- | --- | --- | --- | --- | --- |
| 一级哭声 float | cry | 10784 | 80.80% | 93.64% | 86.74% |
| 一级哭声 INT8 | macro | 10784 | 89.26% | 92.92% | 90.82% |
| 二级哭声 INT8 | 五类 macro | 1018 | 70.68% | 72.13% | 71.13% |
| 场景检测 | infant/head_face | 待补 | 待补 | 待补 | 待补 |
| 遮盖分类 | covered/not_visible | 待补 | 待补 | 待补 | 待补 |
| 行为分类 | edge_grab | 待补 | 待补 | 待补 | 待补 |

# 6. 量化与板端部署

| 模型 | INT8 前后差异 | 执行路径 | 平均/ms | P95/ms | 内存 |
| --- | --- | --- | --- | --- | --- |
| 一级音频 | accuracy 0.925352 -> 0.925723 | CPU INT8, NPU 0% | 31.0 | 31 | 未埋点 |
| 二级音频 | float n=993 与 INT8 n=1018 非同样本 | CPU INT8, NPU 0% | 条件触发 | 待采样 | 未埋点 |
| 场景模型 | 同样本量化差异待补 | Ethos-U command stream | 159 | 159 | arena 784 KiB |
| 遮盖模型 | 同样本量化差异待补 | Ethos-U command stream | 29 | 29 | arena 252 KiB |
| 行为模型 | 同样本量化差异待补 | Ethos-U command stream | 29 | 29 | arena 252 KiB |

# 7. 未执行的实物测试

按用户要求，本轮不再执行需要额外标注集、标准配重、参考温度源、遮挡场景、断网故障注入、Android 配合或长时间运行的测试；这些项目在总表中明确标为“未测试”。

# 8. 原始证据路径

test_results\hardware_20260728_005404\summary.json

test_results\audio_20260728_005959\summary.json

test_results\command_20260728_005355\command_evidence.json

test_results\system_20260728_005433\summary.json

test_results\live_audit_20260728_011600.json

test_results/server_unittest_20260728.txt

test_results/ml_training_pytest_20260728.txt

test_results/vision_training_pytest_20260728.txt

test_results/protocol_contract_20260728.txt

test_results/cpu0_build_20260728.txt

test_results/cpu1_build_20260728.txt